ECU_SOC PROJECT REVIEW

Complete Presentation Script + Speaker Allocation + Faculty Q&A

Presenters

| Person | Primary responsibility in presentation |
| --- | --- |
| Kaushal | Lead presenter + Jatin's system architecture / PID / Ibex / firmware portions |
| Dipiksha | Timing, sensing, calibration, filtering, angle estimation |
| Vedam | Clock/reset, memory, bus, safety, fault monitor |
| Jaydev | PWM, motor interface, FPGA, hardware bring-up |
| Jatin | Not present, but his assigned technical sections are covered by Kaushal |

The actual project responsibility table assigns Jatin system architecture, hardware PID, Ibex integration and firmware; Kaushal I²C and MPU; Dipiksha timing/sensor processing; Vedam memory/bus/clock-reset/safety; and Jaydev PWM/motor/FPGA. ECU-SOC-Project-review-phase-#1

PART 1: HOW TO PRESENT THE PROJECT

Before the actual script, there is one important rule:

Don't pitch this as:

"We are making a self-balancing robot using FPGA."

That makes the project sound like a robotics project.

Instead pitch it as:

"We are designing a small, open ECU as an SoC, where the time-critical control loop is implemented directly in hardware, a RISC-V processor provides supervisory intelligence, and an independent hardware safety layer guarantees deterministic fault response."

Then the robot becomes the demonstrator.

That distinction is crucial.

Your slides themselves establish this positioning: hardware handles sensing-to-PWM, Ibex supervises, and the safety monitor remains independent of software. ECU-SOC-Project-review-phase-#1

PART A

SLIDES 1–4

KAUSHAL

Slide 1: Title

Kaushal:

"Good morning everyone.

Today we are presenting our project, ECU_SOC: a RISC-V hardware ECU implemented as an SoC for a self-balancing robot.

Our teammate Jatin, who leads architecture and integration, is on an internship, so I'll also cover his sections.

Our project sits at the intersection of three areas: embedded control, digital VLSI design, and RISC-V based SoC architecture.

The final objective is not simply to make a robot balance.

Our objective is to design the electronic control unit that makes that balancing decision deterministic, observable, configurable and eventually suitable for an RTL-to-GDSII flow.

So throughout this presentation, we will move from the control problem, to the SoC architecture, to individual RTL blocks, then to FPGA validation and finally to physical design."

Pause.

"I'll begin with the system motivation and architecture, and then each team member will take over the blocks they are responsible for."

Slide 2: What is an ECU?

"First, what exactly do we mean by ECU?

An Electronic Control Unit is essentially an embedded decision-making system.

It receives information from sensors, processes that information within a fixed time constraint, and produces actuator commands.

In a conventional automotive environment, there are many ECUs, for powertrain, braking, body control, HVAC and so on.

Our project takes this general ECU concept and reduces it to a compact research platform.

The plant is a two-wheeled self-balancing robot.

The ECU senses its orientation through an IMU, computes the required correction, and drives two motors.

The interesting part is that we are implementing this ECU itself as an SoC."

Slide 3: Problem Statement

"Now we come to the actual engineering problem.

A two-wheeled balancing robot is an inverted pendulum. If it tilts, the error does not politely wait for software to finish executing.

The controller has to react within milliseconds.

A conventional approach would be a microcontroller running a software PID loop.

That works, but the timing of the control loop becomes dependent on firmware execution, interrupts and processor activity.

Our approach is different.

We move the time-critical loop into dedicated hardware.

The RISC-V processor does not sit inside the control loop. It configures and supervises the hardware.

And if something goes wrong, the safety monitor can disable the motors independently of the CPU."

Then the key line:

"So our architecture separates three responsibilities: hardware controls, the CPU supervises, and safety overrides."

This should become a phrase Kaushal repeats throughout the presentation.

Slide 4: Objectives and Scope

"Our scope has six major parts.

First, synthesizable Verilog RTL for the ECU blocks.

Second, a real-time sensor-to-PWM datapath.

Third, independent hardware safety.

Fourth, integration of the Ibex RV32IMC RISC-V core with memory and a memory-mapped peripheral bus.

Fifth, FPGA validation, first block by block and then as a complete system.

And finally, taking the RTL through a Cadence RTL-to-GDSII flow.

We deliberately keep the first version constrained: single-axis balancing, one IMU and two DC motors.

Features such as LQR, MPC, wireless communication and multi-axis control are future extensions rather than part of the current scope."

Transition

"Before discussing our architecture, we first looked at how this problem is normally approached and where our design fits."

PART B

SLIDES 5–14

KAUSHAL

Slide 5: Concept, Methodology and Positioning

"This section establishes the engineering reasoning behind our architecture."

Don't over-explain. Move quickly.

Slide 6: Inverted Pendulum

"The physical reason this problem is interesting is that the robot behaves approximately as an inverted pendulum.

An upright pendulum is an unstable equilibrium. A small tilt creates a tendency for the error to grow.

In our simplified model, the acceleration of the tilt depends on both the gravitational term and the acceleration generated through the wheels.

For our representative 15-centimetre pendulum length, the characteristic error growth is on the order of a tenth of a second.

That gives us a very important design requirement:

we cannot treat sensing, computation and actuation as independent software tasks. They form one real-time control loop.

The body acts as the pendulum, the wheels provide the movable pivot, the motors generate acceleration and the IMU measures the tilt."

Slide 7: Balancing Loop

"The control loop operates at approximately 512 hertz, meaning one control opportunity every roughly 1.95 milliseconds.

The sensor read consumes roughly 0.4 milliseconds.

The hardware processing itself is targeted at under 5 microseconds.

The remaining latency is dominated by the sensor's internal filtering.

This is where our hardware architecture becomes useful.

Instead of allowing the CPU to determine when every stage runs, we structure the datapath so that every stage progresses according to the clock and valid signals."

Slide 8: Why PID

"We considered several control strategies.

PID is not necessarily the most sophisticated controller here, but it is a very good engineering choice for the first SoC.

It requires only angle and rate information, is computationally compact, maps naturally to fixed-point arithmetic and is straightforward to verify.

LQR and MPC are interesting future directions, but they introduce additional model and computational requirements.

More importantly, our research question is not 'Can we invent a better control law?'

Our question is:

Can we architect a transparent ECU where the time-critical control law executes deterministically in hardware while an open RISC-V processor provides supervisory control?"

That line is excellent for the review.

Slide 9: Sensing and Angle Estimation

"For sensing, we use the MPU-6050.

The accelerometer provides an estimate of tilt, while the gyroscope provides angular rate and therefore a natural derivative input.

Accelerometer-only estimation is noisy under acceleration.

Gyroscope-only estimation is smooth but drifts.

A complementary filter can combine them, while a moving average provides a simpler baseline.

For the first implementation, we prioritize the simpler architecture because every block must be synthesizable and verifiable."

Slide 10: Why Hybrid FPGA + RISC-V

This is one of Kaushal's strongest slides.

"This slide is really the architectural argument for the entire project.

We compared four approaches.

A software PID on an MCU is easy, but timing and safety remain software-dependent.

A fixed-function FPGA gives deterministic timing, but loses flexibility.

A soft CPU doing everything brings us back to software-dependent timing.

So we chose a hybrid architecture.

The hardware datapath handles the deterministic loop.

The RISC-V processor handles configuration, monitoring and higher-level decisions.

The safety logic remains independent.

We accept more design effort in exchange for fixed timing, visibility and safety independence."

Slide 11: Literature Survey

Don't read every row.

"Our literature survey gave us three main observations.

First, FPGA-based control can provide deterministic timing.

Second, PID and complementary filtering are well-established approaches for balancing robots.

Third, RISC-V is increasingly appearing in automotive and control architectures.

However, the works we examined generally focus on one part of this problem.

Some use external MCUs.

Some use commercial FPGA platforms.

Some focus on the control algorithm.

Others focus on RISC-V automotive platforms.

Our project combines these ideas into a small, open and documented reference architecture."

Slide 12: Is Anyone Else Doing Something Similar?

"We should be careful about the word 'novel' here.

The individual concepts are not novel.

Industry already uses the basic pattern of dedicated control hardware plus supervisory processors.

TI has control-law accelerators.

RISC-V motor-control architectures exist.

Automotive RISC-V controllers are being developed.

So we are not claiming to have invented hardware acceleration for control.

Our contribution is more specific:

an open, student-scale reference architecture combining a RISC-V supervisor, hardware PID, CPU-independent safety, FPGA validation and a documented path towards physical design."

This wording will protect you from a professor saying, "But this already exists."

Slide 13: What is Different

"Our differentiation therefore comes from the combination.

The CPU is outside the real-time loop.

Safety can disable the motors independently.

The derivative term comes directly from gyro rate.

The PID contains anti-windup.

Parameters are register-configurable rather than requiring resynthesis.

And the whole architecture uses a frozen interface contract so that five people can develop different blocks in parallel.

Finally, we are taking the design all the way from RTL and simulation to FPGA validation and eventually layout."

Slide 14: Trade-offs

"There is a deliberate trade-off.

A software PID is easier to modify.

Our architecture requires more RTL and verification effort.

But in exchange, we get fixed timing, CPU-independent safety and complete visibility of the datapath.

So we are not claiming our architecture is universally better.

We are saying it is better suited to a deterministic, safety-conscious and inspectable control loop."

Transition

"With that motivation established, I'll now hand over the sensor and timing side of the design."

PART C

SLIDES 15–25

DIPIKSHA

Slide 15

"I'll now take you through the real-time sensing side of the ECU, starting from the system architecture and moving towards the actual sensor-processing blocks."

Slide 16: System Architecture

Point at each block.

"At the highest level, the system has three logical planes.

The first is the sensing and control datapath.

The second is the RISC-V supervisory plane.

And the third is the safety plane.

The IMU enters through the I²C master.

The measurements are captured, calibrated, converted and filtered.

The resulting angle and rate go to the PID accelerator.

The PID command drives PWM, which drives the two motor interfaces.

Parallel to this, Ibex configures the system through memory-mapped registers.

And the safety monitor can override the motor enable independently."

Slide 17: Data Flow, Timing and Conventions

"All of these blocks operate in a single 50 MHz synchronous clock domain.

Each stage communicates using a one-cycle valid pulse.

We use signed Q16.16 fixed-point representation.

For example, 0.25 radians corresponds to 16,384 in this representation.

The important design rule is that interfaces are frozen: signal names, widths, formats and timing conventions are agreed before parallel implementation.

This is essential because five people are developing different blocks."

Slide 18: I²C Master

"The MPU-6050 communicates using I²C, so the first hardware block is our custom I²C master.

The key point is that SDA and SCL are open-drain signals. We don't actively drive them high. We either pull them low or release them.

At 50 MHz, the selected divider gives approximately 390 kilohertz.

Internally, the master uses a bit-level FSM handling START, data bits, ACK, repeated START and STOP.

We also include synchronizers, clock-stretching support and stuck-bus recovery using nine clock pulses.

We verify this against a behavioural slave model before moving to FPGA."

Slide 19: I²C Sensor Subsystem

"Here we can see how the I²C master becomes part of the larger sensor subsystem.

The master does not simply send arbitrary bytes.

The MPU controller requests a defined transaction, receives the burst data and only publishes the sample after the transaction completes correctly.

This gives the rest of the datapath a clean sample-valid boundary."

Slide 20: MPU-6050 Controller

"The controller first performs a startup sequence.

It waits for the device, checks WHO_AM_I for 0x68, wakes the sensor and programs the required configuration.

During normal operation, every control tick triggers a 14-byte burst read beginning at register 0x3B.

That gives us acceleration, temperature and gyro data.

The six sensor values are published together only after STOP, so the downstream pipeline never sees a partially updated sample.

If we receive a NACK or timeout, the transaction is aborted and counted. Three consecutive failures trigger reinitialisation."

Slide 21: Timer, Clock and Reset

"The control scheduler is based on a 32-bit counter.

At 50 MHz, approximately 97,656 cycles gives our 512-hertz control tick.

The timer produces a one-cycle control_tick pulse.

It also generates a much slower CPU interrupt approximately every 51 control ticks, around 10 hertz.

The clock comes from the board oscillator through the MMCM.

Reset asserts immediately but is released synchronously after clock lock.

This gives us one clean timing domain for the control system."

Slide 22: Measurement Capture and Calibration

"The capture stage latches all six axes at the same instant.

This matters because we don't want acceleration and gyro measurements from different sampling moments entering the same control calculation.

Calibration then removes the gyro bias.

We discard the first 64 samples, average the following 256, and subtract the measured bias from subsequent readings.

The safety monitor also prevents arming until calibration has completed."

Slide 23: Calibration

"The calibration FSM is intentionally simple: idle, settle, accumulate and done.

The important physical reason for calibration is that even a small constant gyro offset integrates over time.

A tiny bias therefore becomes a large angle error if left uncompensated.

We also support a hardware-only test mode where calibration can be started without the CPU."

Slide 24: Angle Conversion

"The raw sensor data is converted into the fixed-point format used throughout the datapath.

For the accelerometer, our selected scaling lets us implement the conversion efficiently.

For gyro rate, we use a shift-add approximation rather than a general multiplier.

We also include mounting-dependent axis selection, sign inversion and balance-point trim.

Finally, the angle is compared against the setpoint using saturating subtraction, while gyro rate is passed directly towards the PID derivative term."

Slide 25: Digital Filter

"The baseline filter is a moving average.

We use a circular buffer and running sum, meaning each new sample requires one addition, one subtraction and a shift.

This avoids multipliers.

We support filter lengths from bypass up to a small window.

The trade-off is straightforward: more smoothing gives more noise reduction, but also introduces more phase delay.

We also leave room for a shift-only complementary filter as an optional extension."

Transition

"The filtered angle and rate now become the inputs to the hardware controller. Vedam will take over the system infrastructure and safety architecture, while the control datapath itself will be discussed next."

PART C CONTINUED

SLIDES 26–39

VEDAM + JAYDEV + KAUSHAL

Here we should deliberately split technical ownership rather than forcing one person to present unrelated material.

Slides 26–28: PID

KAUSHAL

Because these belong to Jatin's responsibility.

Slide 26

"I'll take the PID accelerator because this is one of the central architectural blocks of the project.

The important point is that this is not a PID implemented as a software function.

It is a dedicated hardware accelerator.

It accepts the error and gyro rate, performs the P, I and D calculations, sums them and saturates the result before producing the motor command."

Slide 27

"The proportional term uses the current error.

The integral term uses a clamped accumulator.

The derivative term uses the measured gyro rate rather than numerically differentiating noisy angle measurements.

Instead of implementing three separate multipliers, we share one 32-by-32 multiplier through a small state machine.

That reduces hardware cost while keeping the latency within our target.

All gains, limits and setpoints are registers, meaning the CPU can tune the controller without resynthesising the RTL."

Slide 28

"The anti-windup strategy has two rules.

First, the integrator is clamped to a defined limit.

Second, if the output is already saturated in the same direction as the error, the integrator is frozen.

When safety disables the controller, the output goes to zero, the integrator is cleared and the previous error is reset.

This also prevents a derivative or accumulated-error spike when the controller is re-enabled."

Slides 29–31

JAYDEV

Slide 29

"Once the PID generates the signed motor command, the PWM block converts that command into the physical signals required by the motor drivers."

Slide 30

"We selected a 20-kilohertz PWM frequency.

With a 50-megahertz clock, this gives 2,500 clock counts per PWM period.

Positive commands are mapped to RPWM and negative commands to LPWM.

The architecture explicitly prevents both direction outputs from being active simultaneously.

We also implement a five-microsecond dead time on direction changes.

Safety gating is independent of the normal PWM command, so when motor enable is removed, outputs are forced low within the specified two-clock-cycle target."

Slide 31

"Each motor uses one BTS7960 module.

The FPGA provides RPWM, LPWM, R_EN and L_EN.

Direction inversion is configurable so that wiring differences don't require RTL changes.

The motor supply is kept separate from the logic supply, while all logic grounds are common.

During bring-up, we first verify the logic and waveforms with the motors disconnected.

Then we test with the wheels off the ground before allowing full mechanical operation."

Slides 32–34

VEDAM

Slide 32

"I'll now cover the safety architecture.

Safety is deliberately not implemented as a software routine.

It is a hardware monitor sitting outside the normal control loop."

Slide 33

"The safety FSM has three states: DISARMED, ARMED and FAULT.

After reset, the system is disarmed.

To enter ARMED, calibration must be complete, there must be no active or stored fault, and a valid arm request must occur.

Any fault moves the system to FAULT.

Re-arming requires the fault to be cleared and the arm request to transition deliberately.

This prevents an ECU from automatically restarting a motor after a dangerous condition."

Slide 34

"We monitor six major fault classes.

Excessive tilt.

Sensor timeout.

Repeated I²C failures.

Persistent PID saturation.

CPU watchdog timeout.

And the external emergency stop.

Importantly, the motor shutdown path does not wait for software.

The target is removal of motor enable within two 50-megahertz clock cycles, which is 40 nanoseconds.

The CPU is informed through an interrupt, but it is not responsible for stopping the motor."

This is an excellent place for a deliberate pause.

Slides 35–37

VEDAM

Slide 35

"The CPU needs a predictable memory system to configure these hardware blocks.

So we keep the memory architecture deliberately small."

Slide 36

"Ibex has separate instruction and data interfaces.

Instruction fetches go directly to the 8-kilobyte ROM.

Data accesses go through the address decoder to ROM, RAM and our peripheral register banks.

The bus is zero-wait-state in the intended implementation: the request is granted in the same cycle and the response arrives in the next.

An unmapped address returns an error rather than hanging the CPU."

Slide 37

"The memory map gives each hardware function its own 4-kilobyte region.

Timer, I²C, sensor pipeline, PID, PWM and safety each have a defined address range.

Each bank begins with an identification register, so firmware can verify that the hardware is actually connected.

This is particularly useful during bring-up.

We also have a HW_ONLY mode, allowing the real-time datapath to be tested before Ibex is integrated."

Slide 38

KAUSHAL

"This slide shows the overall SoC hierarchy.

The key architecture is that ecu_soc_top owns the integration.

Individual blocks remain modular.

That lets us simulate and validate each block independently before integration, while maintaining a single frozen interface contract."

PART D

SLIDES 39–47

KAUSHAL

Slide 39

"I'll now move to the CPU side of the SoC.

The processor is not the controller of the balance loop.

It is the supervisor of the controller."

Slide 40: CPU / Hardware Boundary

This is a key slide.

"This slide captures one of the most important design decisions in our project.

Ibex performs boot, configuration, monitoring, telemetry, interrupt handling and higher-level behaviour.

Hardware performs sensor acquisition, filtering, PID, PWM and immediate safety response.

Therefore, even if the CPU becomes busy, takes an interrupt or crashes, the hardware control loop retains its timing.

The watchdog can detect the CPU failure separately."

Then the punchline:

"We are not removing software from the ECU. We are removing software from the part of the ECU where timing cannot be allowed to drift."

That's a strong line. Use it.

Slide 41: Core Selection

"We evaluated several open and proprietary processor options.

We selected Ibex because it gives us an open RV32IMC implementation, SystemVerilog RTL, a clean memory interface, a mature verification ecosystem and a configuration small enough for our intended supervisor role.

We don't need a high-performance application processor.

We need a compact, understandable control-plane CPU."

Slide 42: Ibex Overview

"Ibex is a small, in-order 32-bit RISC-V core.

Our selected architecture uses the RV32IMC ISA and a two-stage pipeline.

The integer extension provides the basic instruction set.

M provides multiply and divide.

C provides compressed instructions, which helps reduce code size.

It also supports machine-mode interrupts and a simple request/grant memory interface."

Slide 43: Small Configuration

"We are specifically using the published small configuration as our starting point rather than inventing an arbitrary processor configuration.

The intent is to keep the supervisor compact.

We disable features that are not required for our use case, such as instruction cache and other optional extensions.

Since the PID is implemented in hardware, the CPU doesn't need high arithmetic performance.

Its job is mainly configuration, supervision and firmware execution."

Slide 44: Ibex Architecture

"Internally, Ibex has two major pipeline stages.

Instruction fetch contains the program counter, instruction interface and prefetch structure.

Decode and execute contains the decoder, controller, operand selection, ALU and multiplier/divider.

The register file contains x0 through x31.

The load/store unit is the path through which the CPU accesses our memory and peripheral registers.

CSRs handle machine-mode status, interrupts, traps and counters."

Slide 45

"The important architectural boundary here is the LSU.

Ibex doesn't need to know that an address corresponds to PID or PWM.

It simply performs a load or store.

The bus decoder translates that address into the corresponding hardware register bank."

Slide 46: Memory-Mapped I/O

"For example, firmware can write Kp, Ki and Kd to the PID register bank.

Ibex generates a data request.

The address decoder selects the PID bank.

The value is written into the hardware register.

The next control sample uses that value.

This is what allows us to tune the hardware controller without modifying the datapath itself."

Then:

"Timer and safety events can also interrupt Ibex through the core's interrupt inputs."

Slide 47: Firmware

"The firmware boot sequence is deliberately conservative.

First we initialise memory.

Then we verify all peripheral IDs.

We configure the timer and I²C.

We initialise the sensor and perform calibration.

Then we load PID parameters and safety limits.

The output limit initially starts low.

Only after the system is known to be healthy do we arm the controller.

During normal operation, firmware kicks the watchdog, records telemetry and handles interrupts."

PART E

SLIDES 48–50

DIPIKSHA

Slide 48

"At this point, the architecture is complete enough to ask where this design pattern can be applied."

Slide 49

"The immediate application is the self-balancing robot.

But the architecture is not tied to that robot.

The same pattern can apply to posture control in legged robots, chassis stabilisation in mobile robots, motor-control ECUs and even attitude control in drones and gimbals.

The common requirement is a control loop where deterministic timing and safe actuator shutdown matter."

Slide 50

"Because the hardware is register-configured, we can extend the architecture without redesigning the whole SoC.

We could add wheel encoders, additional sensors, replicated PID channels, outer speed or position loops, CAN or UART, or a more advanced estimator such as a Kalman filter.

The two-wheeled robot is therefore our demonstrator, not the boundary of the architecture."

PART F

SLIDES 51–54

JAYDEV

Slide 51

"I'll now move from the architecture into the actual hardware and toolchain used to realise and validate it."

Slide 52: Hardware Setup

"Our FPGA platform is the Arty A7-100T.

The MPU-6050 connects over I²C.

Two BTS7960 modules receive the PWM and enable signals and independently drive the two DC motors.

Motor power is supplied separately from the logic supply.

We maintain a common ground between the logic and driver side.

An arm switch, E-stop and status LEDs provide physical interaction and safety."

Slide 53: Hardware Specifications

"These specifications define the physical limits of the demonstrator.

The Arty provides the FPGA fabric and interfaces.

The MPU-6050 gives us six-axis sensing.

The BTS7960 modules provide the motor-driving stage.

We are deliberately not claiming a particular FPGA utilisation yet.

Actual utilisation, timing and area numbers will be reported after implementation rather than assumed in advance."

That last sentence is important because your slide explicitly says actual utilisation will be reported after implementation. ECU-SOC-Project-review-phase-#1

Slide 54: Software and EDA Tools

"The toolchain follows the complete lifecycle.

Python provides golden models and analysis.

Vivado handles RTL simulation and FPGA implementation.

ILA and VIO provide on-chip debug.

The RISC-V GNU toolchain builds firmware.

Verilator and Git support code quality and regression.

And finally, Cadence Genus and Innovus provide the path from RTL to synthesized gates and physical layout."

PART G

SLIDES 55–59

VEDAM + JAYDEV

Slide 55

Vedam:

"The next question is how we intend to prove that the architecture actually works."

Slide 56: Development Workflow

Vedam:

"Our development methodology is gate-based.

A block is first implemented against the frozen interface.

It is simulated independently.

Then it is tested on FPGA.

Only after it passes do we integrate it with another block.

The same process continues until the complete ECU is assembled.

RTL-to-GDSII only begins after functional validation.

This prevents a bug from being carried blindly through the entire flow."

Slide 57: Simulation Strategy

Vedam:

"Verification is hierarchical.

Level one tests each block independently.

Level two connects neighbouring blocks.

Level three closes the entire hardware datapath.

Level four introduces the actual Ibex processor and firmware.

We also use assertions for architectural guarantees.

For example, RPWM and LPWM must never be high simultaneously.

We test reset behaviour, saturation, boundary conditions, bad addresses and injected I²C failures."

Slide 58: FPGA Test Plan

Jaydev:

"The FPGA validation follows four stages.

Stage A tests each block independently.

Stage B combines the blocks progressively.

Stage C tests the full hardware control loop without the CPU.

Stage D runs the complete ECU with Ibex and firmware.

We also deliberately test extreme conditions: noisy sensors, saturation, reset during operation, unplugged sensors, E-stop and watchdog faults.

Each test will have defined pass criteria and logged results."

Slide 59: RTL-to-GDSII

Vedam:

"Once the design is functionally stable, we move into physical design.

We freeze the RTL, lint it and define timing constraints.

Genus synthesizes the RTL into a gate-level netlist.

The netlist is simulated.

Innovus then performs floorplanning, placement, clock-tree synthesis and routing.

Finally, timing, power and physical checks are performed before exporting GDSII.

We have deliberately avoided FPGA-specific primitives inside the core RTL so that the design remains portable towards this stage."

PART H

SLIDES 60–63

SHARED FINAL TEAM SECTION

Here I recommend rapid handoffs.

Slide 60: Timeline

Kaushal

"Our timeline is divided into three major phases.

First, RTL development and simulation from October to November.

Second, FPGA validation from November through January or early February.

Third, RTL-to-GDSII from February to May.

The CPU track runs in parallel because processor integration should not become a late-stage dependency."

Slide 61: Timeline Details

Dipiksha

"The key principle is that these dates are flexible, but the gates are not.

We do not move forward simply because the calendar says so.

RTL must be ready.

FPGA validation must pass.

Only then does the design proceed to layout."

Slide 62: Team Responsibilities

Kaushal

This is a good place to explicitly explain your absence without making it awkward.

"Our work is divided by architectural blocks.

Jatin is leading the system architecture, hardware PID, Ibex integration and firmware.

Since Jatin is currently away for his internship, I am also taking over his presentation responsibilities today.

Kaushal, myself, handle the I²C and MPU subsystem.

Dipiksha handles timing, measurement and filtering.

Vedam handles memory, bus, clock/reset and safety.

Jaydev handles PWM, motor interface, FPGA wrapper and motor bring-up.

We maintain a common interface contract and integrate through Git reviews and weekly checkpoints."

Don't say:

"Jatin couldn't come so I have to do his part."

Say:

"Jatin is currently away for his internship, so I am covering his presentation sections in addition to my own."

Much more professional.

Slide 63: Risks

Everyone can take one risk.

Kaushal:

"For schedule risk, we use gate-based development and prioritise the essential features."

Dipiksha:

"For sensor noise and latency, filter parameters remain configurable and the complementary filter remains an optional extension."

Vedam:

"For safety bugs, the monitor is independent, fault cases are tested individually and in combination, and the logic is peer reviewed."

Jaydev:

"For hardware bring-up, we start with logic-only tests, then driver tests with the wheels off the ground, before allowing full operation."

Then Kaushal:

"And for integration risk, our main mitigation is the frozen interface contract."

Slide 64: Expected Outcomes and Conclusion

KAUSHAL

Slow down here.

"So, what do we expect to deliver?

First, a verified library of ECU RTL blocks and a complete SoC.

Second, an FPGA demonstration of the hardware control loop on the two-wheeled robot, with measured timing, latency and fault-response behaviour.

Third, physical design results from the RTL-to-GDSII flow.

And fourth, documentation that makes the architecture reusable by others."

Then the strongest conclusion:

"The central idea of our project can be summarised in one sentence:

The balance controller is hardware, the RISC-V processor is the supervisor, and safety is independent of software.

We believe this gives us a useful middle ground between a purely software-controlled embedded system and a completely fixed-function controller.

It gives us deterministic timing without giving up configurability.

It gives us hardware safety without making the entire system inflexible.

And because the design is open and modular, we can take it from simulation, to FPGA, and ultimately towards silicon."

Pause.

"That is the architecture we are proposing."

Slide 65: References

Jaydev

"These are the principal references supporting the control, FPGA, RISC-V, driver and hardware choices made in our design."

Don't read references.

Slide 66: Thank You

Kaushal

"Thank you for listening.

We would be happy to discuss the architecture, the individual RTL blocks, the control strategy, verification methodology or the RTL-to-GDSII path."

Then stop talking.

Don't immediately say "Any questions?"

Let the panel begin.

VERY IMPORTANT

HOW KAUSHAL SHOULD HANDLE JATIN'S ABSENCE

Do not apologise repeatedly.

At the beginning:

"Jatin is currently away for his internship, so I'll also be covering his system-architecture, PID and CPU-integration sections."

That's enough.

If asked specifically about Jatin:

"Jatin is currently continuing the implementation work remotely. His primary responsibilities are system integration, hardware PID, Ibex integration and firmware, so I have covered those presentation sections today."

That sounds like a functioning team, not an absent member.

PART 2

PROFESSOR QUESTIONS

This is where I would prepare very seriously.

Your professors are likely to attack the architecture from four directions:

- What is actually novel?

- Why hardware instead of software?

- Are your timing/safety claims physically believable?

- Can this actually become silicon?

Below are the answers I would prepare.

Q1. "What exactly is novel in your project?"

Answer

"The individual components are not novel. PID, FPGA control, RISC-V and hardware safety are all established.

Our contribution is the open integration of these ideas into a small ECU SoC where the time-critical sensor-to-PWM loop is implemented independently of the CPU, safety is independent of software, configuration is memory-mapped, and the entire design is documented from RTL through FPGA validation towards GDSII.

So our novelty is architectural and integrational rather than inventing a new PID algorithm."

If they push:

"We are deliberately not claiming industrial-level novelty. We are building a transparent reference implementation of a proven automotive-style architectural pattern at student scale."

This is probably your safest answer.

Q2. "Why not just use a microcontroller?"

Answer

"We absolutely could, and that is a valid engineering solution.

But our objective is to investigate deterministic hardware control and SoC architecture.

In a software MCU solution, loop execution timing depends on instruction execution, interrupts and firmware scheduling.

In our architecture, the control pipeline progresses according to the hardware clock, while the CPU remains available for supervision.

So the trade-off is higher implementation complexity in exchange for deterministic latency and CPU-independent safety."

Q3. "Why do you even need RISC-V if hardware is doing everything?"

Answer

"Because not every ECU function needs hard real-time hardware.

Hardware handles the repetitive, deterministic control loop.

The CPU handles configuration, calibration management, telemetry, fault logging, watchdog service, interrupts and future higher-level behaviour.

So the CPU gives us flexibility without putting the timing-critical loop under software control."

Excellent answer.

Q4. "If the CPU crashes, what happens?"

Answer

"The real-time balance loop continues because it is implemented entirely in hardware.

The watchdog detects the lack of CPU service.

If the watchdog is enabled and expires, the safety monitor latches a fault and disables the motor output.

So a CPU failure does not directly disrupt the deterministic datapath, but it can still result in a safe shutdown."

Q5. "Why not put the safety logic in the CPU?"

Answer

"Because then the safety response would depend on the CPU being operational and correctly executing the software.

Our design intentionally removes that dependency.

The safety monitor continuously observes the relevant conditions and directly controls the motor-enable path.

The CPU receives the fault as an interrupt for logging and recovery, but it is not responsible for the initial motor shutdown."

Q6. "You claim 40 ns fault response. Is that realistic?"

This is a trap question.

Answer:

"The 40-nanosecond figure is the digital hardware response target from the fault signal reaching the safety logic to the motor-enable gating decision, assuming the fault is already represented synchronously within the FPGA clock domain.

It does not mean the physical motor stops in 40 nanoseconds.

Driver propagation, motor inertia and mechanical stopping time are much longer.

The claim is specifically about deterministic digital fault response."

Excellent distinction.

Q7. "Why 512 Hz?"

Answer

"It gives us a control opportunity every approximately 1.95 milliseconds.

That is comfortably within the timescale needed for the balancing demonstrator while giving the I²C transaction sufficient time to complete.

We also keep the architecture configurable so the exact control rate can be tuned during FPGA validation."

Q8. "Why 20 kHz PWM?"

Answer

"It is above the audible range and within the BTS7960's specified PWM capability.

With a 50 MHz clock, 20 kHz gives exactly 2,500 clock counts per period, which is convenient for digital implementation and gives adequate duty resolution."

Q9. "Why Q16.16?"

Answer

"It gives us a practical balance between range and fractional resolution.

We need signed quantities such as angle, rate and control effort, while keeping the datapath synthesizable and reasonably compact.

It also makes scaling and verification deterministic compared with floating-point arithmetic."

Q10. "Why not floating point?"

Answer

"Floating point would increase hardware complexity and is unnecessary for this control problem.

The required dynamic range and precision can be represented using fixed-point arithmetic.

Fixed-point also makes latency and resource usage much more predictable."

Q11. "Why use the gyro rate directly for D instead of differentiating angle?"

Answer

"Numerical differentiation amplifies noise.

The gyroscope already directly measures angular rate, so using that measurement provides a more natural derivative input and avoids introducing another differentiator into the datapath."

Q12. "Why PID instead of LQR?"

Answer

"LQR is a valid future direction, but it requires a more complete state model and typically additional state measurements.

Our current demonstrator focuses on a single-axis balance loop using the IMU.

PID gives us a compact, interpretable and fixed-point-friendly hardware controller that is much easier to verify as the first implementation.

Once the hardware architecture is validated, an LQR accelerator could be a future extension."

Q13. "Why not Kalman filtering?"

Answer

"Kalman filtering can provide better state estimation, but it requires substantially more arithmetic and model complexity.

Our first objective is to validate the deterministic ECU architecture.

We therefore use a lightweight filtering approach initially and leave Kalman filtering as a future hardware block."

Q14. "What happens if the IMU is unplugged?"

Answer

"The sensor transaction eventually times out or fails.

The I²C error counter and sensor timeout logic detect this condition.

After the configured threshold, the safety monitor latches a fault and removes motor enable.

The CPU is informed through the fault interrupt, but again, software isn't required for the initial shutdown."

Q15. "What happens if the sensor gives one bad sample?"

Answer

"We don't want a single transient sample to unnecessarily shut down the robot.

Therefore several safety conditions use consecutive-sample or persistence requirements.

For example, the tilt fault requires three consecutive samples beyond the threshold.

The verification plan explicitly includes glitch-rejection tests."

Q16. "What happens if PID saturates?"

Answer

"The output is saturated at the configured limit.

The integrator is prevented from continuing to wind up.

If saturation persists for the configured number of samples, the safety monitor treats it as a fault condition and disables the motors."

Q17. "Why do you need anti-windup?"

Answer

"Without anti-windup, if the motor command is saturated, the integral term can continue accumulating error.

When the system eventually leaves saturation, that stored integral can cause excessive overshoot and slow recovery.

Our clamping and freeze rules prevent that."

Q18. "Why use one shared multiplier instead of three?"

Answer

"Because the PID terms do not need three multiplications simultaneously.

We time-share one 32-by-32 multiplier through a small FSM.

This reduces area at the cost of a few cycles, while remaining comfortably inside our latency target."

Q19. "Why is the CPU's multiplier not used for PID?"

Answer

"That would put the control loop back under processor execution.

Our architectural objective is that the real-time path should not depend on CPU availability.

Therefore the PID has dedicated arithmetic."

Q20. "What if the CPU changes the PID gain while the controller is running?"

Answer

"The gain registers are memory-mapped and can be updated by software.

In the final implementation, we need to define and verify the register update boundary so a gain change does not create a partially updated control calculation.

The preferred approach is to latch configuration values at a defined control-sample boundary."

This is a good answer because it doesn't pretend an unspecified detail is already solved.

Q21. "Why does every register bank have an ID?"

Answer

"It provides a simple hardware-software integration check.

During boot, firmware can read each bank's ID and verify that the expected peripheral exists at the expected address.

It makes bring-up and debugging much easier."

Q22. "Why is there no cache?"

Answer

"The CPU is a supervisory processor rather than a performance processor.

A cache would add area and timing complexity without providing significant benefit for our workload.

Disabling it also makes memory behaviour more predictable."

Q23. "Why Ibex?"

Answer

"It is open, SystemVerilog-based, well documented, RISC-V compliant, relatively small and has a clean memory interface.

It also has a mature verification ecosystem and is already used in OpenTitan.

Those characteristics make it suitable both for FPGA experimentation and eventual ASIC-oriented exploration."

Q24. "Why not PicoRV32?"

Answer

"PicoRV32 is very compact and was considered.

We selected Ibex because the project benefits from its active maintenance, documentation, RV32IMC configuration and clearer path for the specific integration and verification requirements we have."

Don't criticise PicoRV32 aggressively.

Q25. "Why not MicroBlaze?"

Answer

"MicroBlaze is tied to the FPGA-vendor ecosystem.

Our long-term objective includes an RTL-to-GDSII flow.

We therefore need a processor whose RTL can move beyond the FPGA vendor environment."

Q26. "Can your design really go to ASIC?"

Answer

"The architecture is designed with that goal in mind.

The core RTL avoids FPGA-specific primitives.

We use a single synchronous clock domain.

Memories are parameterised.

The design is synthesizable RTL.

After FPGA validation, we plan to take the same RTL through Genus and Innovus.

However, we will distinguish clearly between what has been validated on FPGA and what has actually been demonstrated in the ASIC flow."

That last sentence is very important.

Q27. "Why FPGA first?"

Answer

"Because FPGA gives us a practical way to validate real interfaces, timing, I²C behaviour, PWM outputs, safety responses and the actual robot before committing to physical design.

It is much cheaper to discover architectural errors before synthesis and layout."

Q28. "What exactly are you going to measure?"

Answer

"We intend to measure control-loop period, sensor-to-PWM latency, timing jitter, FPGA resource utilisation, maximum operating frequency, safety fault response, and functional behaviour under injected faults.

These are measurements rather than assumptions."

Q29. "How will you prove that your PID is correct?"

Answer

"At block level, we compare the RTL against a Python golden model using directed and random vectors.

We test proportional, integral and derivative behaviour, saturation, anti-windup and fixed-point edge cases.

Then we integrate it with the sensor pipeline and PWM block and finally test it in the closed-loop FPGA system."

Q30. "How will you verify safety?"

Answer

"Every fault is tested individually and in combination.

We test tilt, sensor timeout, I²C failure, PID saturation, watchdog failure and E-stop.

We also test glitch rejection, reset during operation and deliberate re-arm behaviour.

Assertions will verify properties such as motor outputs being disabled in FAULT and RPWM/LPWM never being active simultaneously."

Q31. "What if the safety monitor itself has a bug?"

This is a harder question.

Answer:

"That's why safety logic receives dedicated verification rather than relying only on system-level testing.

We will use independent testbenches, injected fault scenarios, assertions and peer review.

We also keep the safety logic deliberately simple.

Ultimately, if this were a production safety-certified ECU, much more rigorous formal verification and certification would be required. Our current target is a research prototype, not an ASIL-certified automotive product."

Excellent answer.

Q32. "Is this actually an automotive ECU?"

Answer:

"Architecturally, it follows the ECU pattern.

But we are not claiming automotive qualification.

Our demonstrator is a self-balancing robot and our objective is to demonstrate the architecture and implementation methodology at student/research scale."

Never claim automotive qualification.

Q33. "Where exactly is your contribution compared with commercial ECUs?"

Answer:

"Commercial ECUs are proprietary, highly integrated products.

Our contribution is an open and inspectable reference architecture where the complete path from sensor interface, hardware control, CPU supervision and safety to RTL-to-GDSII methodology can be studied and reproduced."

Q34. "Why five people? Is the architecture really modular?"

Answer

"Yes, and the frozen interface contract is deliberately part of the architecture.

Each block has defined inputs, outputs, timing and fixed-point formats.

This allows independent development and block-level verification before integration.

The team structure therefore mirrors the hardware modularity."

Q35. "What is your biggest technical risk?"

Best answer:

"Integration.

Individual blocks are relatively manageable.

The difficult part is making sure timing, fixed-point formats, reset behaviour, valid signals and memory-mapped interfaces all agree when the blocks are connected.

That's why our development process is pairwise integration followed by full-datapath verification rather than directly assembling everything at once."

This is a mature answer.

Q36. "What is the biggest limitation of your current architecture?"

Answer

"The current controller is deliberately limited to a single-axis PID architecture.

That makes the first implementation tractable and deterministic, but it limits the control sophistication.

Multi-axis control, wheel encoders, more advanced estimation and controllers such as LQR or MPC are natural future extensions."

Q37. "What happens if the robot falls?"

Answer

"The tilt condition eventually exceeds the configured threshold.

After the required consecutive samples, the safety monitor latches FAULT and removes motor enable.

The CPU is notified, but software isn't required for the immediate shutdown."

Q38. "Why does the safety monitor need both live and stored faults?"

Answer

"A live condition tells us whether the fault currently exists.

A stored or latched fault tells us that a dangerous event occurred and prevents automatic re-entry into ARMED.

This is important because a transient dangerous event shouldn't simply disappear and allow the robot to restart itself."

Q39. "Why does the E-stop have special treatment?"

Answer

"Because E-stop is a physical safety input and should not depend on the normal software or control pipeline.

It is therefore handled as an immediate safety condition that removes motor enable."

Q40. "What if the I²C bus gets stuck?"

Answer

"The I²C master includes stuck-bus recovery using nine clock pulses.

If the transaction still fails, the error is counted.

Persistent failure eventually reaches the safety layer and causes a motor shutdown."

Q41. "Why 14-byte reads?"

Answer

"The MPU-6050 provides acceleration, temperature and gyro registers as a contiguous data block.

Reading the complete 14-byte frame ensures that all six motion measurements associated with the sample are captured coherently."

Q42. "Why 390 kHz and not exactly 400 kHz?"

Answer

"Our divider is chosen from the 50 MHz system clock and gives approximately 390 kHz.

It remains within the MPU-6050's 400-kHz I²C limit while keeping the digital divider implementation straightforward."

Q43. "What is your actual measured latency?"

Be careful.

Answer:

"At this project-review stage, the values on the slides are design targets or calculated timing estimates where explicitly stated.

We will report measured FPGA latency and jitter after implementation.

We don't want to present a calculated number as though it were already experimentally measured."

This is probably one of the most important answers to memorise.

Q44. "Do you have a working robot already?"

If you don't:

"The current phase is architecture and RTL development. The FPGA validation and robot bring-up are scheduled in the next phase.

Our presentation therefore distinguishes between current design targets and future measured results."

Never bluff.

Q45. "What happens if the filter makes the system too slow?"

Answer

"That is a known trade-off.

Increasing the moving-average window reduces noise but adds phase delay.

That's why the filter length is configurable and deliberately kept small.

During FPGA testing, we will measure the effect on stability rather than assuming that the mathematically smoother signal is always better."

THE 10 QUESTIONS I WOULD EXPECT MOST

If you have limited preparation time, memorise these:

1.

What's novel?

Open combination of hardware real-time loop + RISC-V supervisor + independent safety + FPGA-to-GDSII methodology.

2.

Why FPGA/hardware instead of MCU?

Fixed latency, reduced jitter, CPU-independent control and safety.

3.

Why RISC-V if hardware does the control?

Configuration, monitoring, telemetry, interrupts and future higher-level behaviour.

4.

What if CPU crashes?

Hardware loop continues; watchdog eventually causes safe shutdown.

5.

What if sensor disconnects?

Timeout/error → safety fault → motor enable removed.

6.

Why PID?

Compact, deterministic, fixed-point friendly and appropriate for first implementation.

7.

Why not LQR/Kalman/MPC?

Scope, computational complexity and additional state/model requirements. Future extension.

8.

How do you verify it?

Block → pair → full datapath → CPU → FPGA → physical design.

9.

Is 40 ns motor stopping time?

No. It is digital motor-enable shutdown response, not mechanical motor stopping.

10.

Have you measured the claimed timing?

Not yet where the slides identify targets/estimates. FPGA measurements are part of the next validation phase.

FINAL "KILLER" QUESTION

There is one question I would absolutely expect from a good VLSI professor:

"If the CPU is not in the control loop, why call this an SoC? Isn't it just an FPGA control circuit with a CPU attached?"

"The distinction is that the processor is not an afterthought or an external controller.

It is integrated into the same SoC and connected through a defined memory and peripheral architecture.

The hardware datapath and CPU share the same clock, memory map and configuration space.

The architectural decision is to assign different workloads to different hardware resources.

The deterministic workload goes to dedicated hardware.

The programmable supervisory workload goes to RISC-V.

So the SoC is heterogeneous by design, rather than CPU-centric."

That is probably the single best answer in the entire preparation.

FINAL PRESENTATION CHEAT SHEET

If someone remembers nothing else, remember these five sentences:

1.

"The robot is our demonstrator. The actual project is the ECU SoC."

2.

"Hardware controls, RISC-V supervises, safety overrides."

3.

"We are not claiming novelty in PID, FPGA or RISC-V individually. Our contribution is the open, documented combination."

4.

"The CPU is deliberately outside the time-critical control loop."

5.

"Our timing, latency and area numbers will ultimately be measured, not assumed."

Those five statements give the whole presentation a spine.

And if a professor gets aggressive with the novelty question, don't fight it. Say:

"Yes, the individual ideas are established. We see our contribution as an architectural and implementation reference rather than claiming a fundamentally new control algorithm."

That answer will make the project sound more credible, not less.

The final deck itself supports this positioning across the architecture, CPU boundary, safety, verification and expected-outcome sections. ECU-SOC-Project-review-phase-#1