Processor-Based Real-Time ECU SoC

Block-Level Architecture Documentation

Version 1.0

1. Processor

Purpose

The processor acts as the supervisory unit of the entire ECU. It is responsible for system initialization, peripheral configuration, parameter management, and monitoring. Unlike conventional microcontroller-based systems where the processor executes the complete control algorithm, the processor in this architecture primarily performs system-level management while delegating time-critical computations to dedicated hardware modules.

Responsibilities

- Initialize the entire system after reset.

- Configure all peripheral modules.

- Configure Timer frequency.

- Configure I²C communication parameters.

- Configure Control Module parameters.

- Monitor system status.

- Read sensor information when required.

- Enable or disable peripherals.

Inputs

- Clock

- Reset

- Data from System Bus

Outputs

- Address Bus

- Data Bus

- Read Enable

- Write Enable

- Control Signals

Connected Modules

- Memory

- System Bus

- Timer

- I²C Master

- Control Module

2. System Bus

Purpose

The System Bus is the communication backbone of the ECU. It provides a common communication channel between the processor and every peripheral inside the SoC.

Rather than directly connecting every module to the processor, all communication is routed through the System Bus using memory-mapped addresses.

Responsibilities

- Address decoding

- Data transfer

- Read operations

- Write operations

- Peripheral selection

Inputs

- Address

- Data

- Read Enable

- Write Enable

Outputs

- Selected peripheral

- Returned data

Connected Modules

- Processor

- Memory

- Timer

- I²C Master

- Control Module

3. Memory System

Purpose

The Memory System stores firmware, runtime variables, configuration parameters and processor instructions.

The processor accesses the memory using the System Bus.

Components

ROM

Stores

- Program instructions

- Constant values

- Default parameters

RAM

Stores

- Variables

- Sensor readings

- Runtime data

- Temporary calculations

Responsibilities

- Store processor program

- Store configuration parameters

- Store runtime variables

Inputs

- Address

- Write Data

- Write Enable

Outputs

- Read Data

4. Timer Unit

Purpose

The Timer Unit is responsible for maintaining deterministic execution of the control loop.

Instead of allowing the control algorithm to execute randomly, the Timer periodically generates a control tick which synchronizes the entire closed-loop system.

Responsibilities

- Generate periodic control tick

- Maintain constant sampling frequency

- Synchronize sensor acquisition

- Synchronize control execution

Internal Components

- Prescaler

- Counter

- Comparator

- Tick Generator

Inputs

- System Clock

- Reset

Outputs

- Tick Signal

Connected Modules

- Control Module

- Measurement Register

- Processor (optional interrupt)

5. I²C Master

Purpose

The I²C Master communicates with the external IMU sensor.

It periodically requests motion data from the IMU and transfers the received information into the ECU.

Responsibilities

- Generate START condition

- Send device address

- Read sensor registers

- Receive data

- Generate STOP condition

Inputs

- Tick Signal

- Processor Configuration

Outputs

- Sensor Data

- Status Information

Connected Modules

- IMU

- Measurement Register

- Processor

6. Measurement Register

Purpose

The Measurement Register stores the latest IMU measurement and synchronizes it with the control loop.

This prevents the Control Module from reading unstable or partially updated sensor values.

Responsibilities

- Store IMU data

- Synchronize measurements

- Provide stable data to Error Calculation

Inputs

- IMU Data

Outputs

- Measurement Value

Connected Modules

- I²C Master

- Error Calculation

7. Error Calculation Unit

Purpose

The Error Calculation Unit compares the desired system state with the measured system state.

The resulting error becomes the primary input for the Control Module.

Responsibilities

- Calculate control error

- Provide signed error value

Inputs

- Desired Angle

- Measured Angle

Outputs

- Error

Connected Modules

- Measurement Register

- Control Module

8. Control Module

Purpose

The Control Module is the computational core of the ECU.

It receives the calculated error and determines the corrective control effort required to stabilize the system.

Depending on the final implementation, this module may contain a hardware-accelerated PID controller or another deterministic control algorithm.

Responsibilities

- Process control error

- Generate corrective output

- Maintain system stability

Inputs

- Error

- Control Parameters

- Tick Signal

Outputs

- Control Output

Connected Modules

- Error Calculation

- PWM Generator

- Processor

9. PWM Generator

Purpose

The PWM Generator converts the digital control output into Pulse Width Modulated signals suitable for motor control.

Responsibilities

- Generate PWM waveform

- Control duty cycle

- Maintain fixed switching frequency

Inputs

- Control Output

Outputs

- PWM Left

- PWM Right

Connected Modules

- Motor Interface

10. Motor Interface

Purpose

The Motor Interface converts the PWM signals and control direction into signals required by the external H-Bridge motor driver.

Responsibilities

- Generate Direction signals

- Generate Enable signals

- Forward PWM signals

Inputs

- PWM

- Control Direction

Outputs

- PWM

- Direction

- Enable

Connected Modules

- PWM Generator

- External Motor Driver

11. IMU (External Peripheral)

Purpose

The Inertial Measurement Unit continuously measures the orientation and angular motion of the robot.

The IMU provides the feedback required for closed-loop stabilization.

Measurements

- Acceleration

- Angular Velocity

- Estimated Tilt Angle

Communication Protocol

- I²C

Connected Modules

- I²C Master

12. External Motor Driver

Purpose

The Motor Driver amplifies the low-power digital control signals generated by the ECU into high-current signals capable of driving DC motors.

Responsibilities

- Drive motors

- Direction control

- Current handling

- Power amplification

Inputs

- PWM

- Direction

- Enable

Outputs

- Motor Voltage

- Motor Current

Connected Modules

- Motor Interface

- DC Motors

Complete System Flow

Power ON

│

▼

Processor Initialization

│

▼

Memory Configuration

│

▼

Timer Starts

│

▼

Periodic Tick Generated

│

▼

I²C Reads IMU

│

▼

Measurement Register Updated

│

▼

Error Calculation

│

▼

Control Module

│

▼

PWM Generator

│

▼

Motor Interface

│

▼

Motor Driver

│

▼

DC Motors

│

▼

Robot Motion

│

▼

IMU Measures New Orientation

│

└─────────── Feedback Loop ───────────┘